# TASK-1

TITLE: Using a temperature sensor to read and display temperature data on an LCD or serial monitor

OBJECTIVE: The objective of this project is to design and implement a temperature monitoring system using a temperature sensor and Arduino, and display the temperature readings on a Serial Monitor or LCD.

COMPONENETS: 1. Arduino UNO

2. LM35 Temperature Sensor

3.16×2 LCD Display (optional)

4. Breadboard

5. Jumper wires

6. USB Cable

SOFTWARE USED:  Tinkercad

WORKING PRINCIPLE: The LM35 temperature sensor produces an output voltage that is directly proportional to the temperature in Celsius.

For every 1°C, LM35 gives 10 mV output

Arduino reads this analog voltage using its ADC (Analog-to-Digital Converter)

The voltage is converted into temperature using the formula:

Temperature (°C) = Voltage × 100

The calculated temperature is then displayed on the Serial Monitor or LCD.

CIRCUIT DIAGRAM:

ARDUINO CODE:

#include "LiquidCrystal.h"

LiquidCrystal lcd(8,7,6,5,4,3);

int sensorPin = 0;

void setup()

{

Serial.begin(9600);

lcd.begin(16,2);

}

void loop()

{

int reading = analogRead(sensorPin);

// measure the 5v with a meter for an accurate value

// in particular if your Arduino is USB powered

float voltage = reading * 4.68;

voltage /= 1024.0;

// now print out the temperature

float temperatureC = (voltage - 0.5) * 100;

Serial.print(temperatureC);

Serial.println(" degrees C");

lcd.setCursor(0,0);

lcd.print("Temperature Value ");

lcd.setCursor(0,1);

lcd.print(" degrees C");

lcd.setCursor(11,1);

lcd.print(temperatureC);

delay(100);

}

OUTPUT:

The system continuously reads temperature and displays values like :

- 19.93

- 60.50

PRACTICAL OUTPUT:

RESLUT:

The temperature monitoring system was successfully implemented using Arduino and LM35 sensor. The system accurately measures and displays temperature values on the Serial Monitor/LCD.